Central Coast Council

PFAS Monitoring Results

## August 2024 - August 2025

# Ourimbah Catchment

Figure 1: Sample sites for Ourimbah Creek catchment (north-western sites, excluding Springs Road at far north of catchment).

Figure 2: Sample sites for Ourimbah Creek catchment (south-easten sites)

| Site No | Location |
| --- | --- |
| 1 | Stringybark Creek downstream of Mangrove Mountain Golf Course |
| 2 | Ourimbah Creek most upstream point |
| 3 | Most downstream point of Hallards Creek |
| 4 | Stringybark Creek upstream of Ourimbah Creek confluence (connection) |
| 5 | Mid Ourimbah creek in proximity of 40 Acres |
| 6 | Water Pump Station 11 north / west of Ourimbah interchange of M1 |
| 7 | Northern arm of Stringybark Creek downstream from site 1 |
| 8 | Southern arm of Stringybark Creek upstream from site 4 |
| 9 | Upstream of Stringybark Creek – Mangrove Mountain Golf Course |
| 10 | Mangrove Mountain Golf Course Northern pond |
| 11 | Stringybark Creek upstream of site 9 |
| 12 | Stringybark Creek Southern arm adjacent to Mangrove Mountain Landfill |
| 13 | Northern arm of Stringybark Creek upstream of Mangrove Mountain Landfill |
| 14 | Hallards Creek upstream of site 3 |
| 15 | Stormwater runoff adjacent to Mangrove Mountain Landfill site |
| 16 | Ourimbah Creek upstream from site 17 |
| 17 | Ourimbah Creek upstream from site 6 |

*Sites are ordered from bottom of catchment to the upper reaches of the catchment

** Units are in micrograms per litre

| Site | Month | PFAS Substance | Notes |
| --- | --- | --- | --- |
| 6 | 8 August 2024 | N/D |  |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | N/D |  |
| 7 |  | N/D |  |
| 8 |  | N/D |  |
| 6 | 13 November 2024 | PFHxS (0.002) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.002) PFBS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.006) PFHxS (0.047) PFOA (0.019) PFBS (0.065) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 6 | 12 December 2024 | PFOS (0.002) PFHxS (0.002) PFOA (0.002) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.002) PFOA (0.002) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.007) PFHxS (0.039) PFOA (0.039) PFBS (0.04) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 1 | 15 January 2025 | N/D |  |
| 6 |  | PFHxS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.002) PFBS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.007) PFHxS (0.046) PFOA (0.015) PFBS (0.048) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 9 |  | PFOS (0.033) PFHxS (0.211) PFOA (0.086) PFBS (0.145) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.023) PFHxS (0.157) PFOA (0.074) PFBS (0.121) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 6 | 23 January 2025 | PFOS (0.001) PFHxS (0.003) PFOA (0.001) PFBS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.003) PFOA (0.001) PFBS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 6 |  | N/D |  |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.007) PFHxS (0.039) PFOA (0.018) PFBS (0.039) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 9 |  | PFOS (0.061) PFHxS (0.254) PFOA (0.139) PFBS (0.175) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.031) PFHxS (0.163) PFOA (0.093) PFBS (0.134) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.051) PFHxS (0.297) PFOA (0.168) PFBS (0.22) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.068) PFHxS (0.234) PFOA (0.197) PFBS (0.168) | PFOA and PFBS below both current Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 14 |  | N/D |  |
| 15 |  | PFOS (0.005) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 6 | 6 February 2025 | PFHxS (0.002) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.002) PFOA (0.001) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.01) PFHxS (0.042) PFOA (0.019) PFBS (0.031) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 9 |  | PFOS (0.021) PFHxS (0.168) PFOA (0.076) PFBS (0.105) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.038) PFHxS (0.207) PFOA (0.119) PFBS (0.119) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.018) PFHxS (0.151) PFOA (0.075) PFBS (0.103) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.111) PFHxS (0.74) PFOA (0.417) PFBS (0.57) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 14 |  | N/D |  |
| 15 |  | PFOS (0.007) PFHxS (0.011) PFOA (0.007) PFBS (0.01) | PFOS below current Australian Drinking Water Guidelines only. PFHxS, PFOA and PFBS detection below both current and proposed Guidelines |
| 6 | 13 February 2025 | PFHxS (0.002) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.001) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.01) PFHxS (0.042) PFOA (0.018) PFBS (0.031) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 9 |  | PFOS (0.015) PFHxS (0.138) PFOA (0.065) PFBS (0.096) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.039) PFHxS (0.183) PFOA (0.113) PFBS (0.088) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.012) PFHxS (0.148) PFOA (0.062) PFBS (0.084) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.1) PFHxS (0.643) PFOA (0.351) PFBS (0.217) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 14 |  | N/D |  |
| 15 |  | PFOS (0.005) PFHxS (0.009) PFOA (0.004) PFBS (0.01) | PFOS below current Australian Drinking Water Guidelines only. PFHxS, PFOA and PFBS detection below both current and proposed Guidelines |
| 6 | 18 March 2025 | PFHxS (0.002) PFOA (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 5 |  | PFHxS (0.002) PFBS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.008) PFHxS (0.04) PFOA (0.018) PFBS (0.034) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS below current Guidelines only. |
| 9 |  | PFOS (0.012) PFHxS (0.143) PFOA (0.06) PFBS (0.103) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.032) PFHxS (0.176) PFOA (0.106) PFBS (0.12) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.013) PFHxS (0.135) PFOA (0.063) PFBS (0.105) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.053) PFHxS (0.553) PFOA (0.31) PFBS (0.635) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 14 |  | N/D |  |
| 15 |  | PFOS (0.002) PFHxS (0.003) PFOA (0.002) PFBS (0.007) | Detection below both current and proposed Australian Drinking Water Guidelines. |
| 6 | 9 April 2025 | PFHxS (0.002) PFOA (0.002) | Detection below Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 17 |  | PFHxS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines. |
| 16 |  | PFHxS (0.001) | Detection below both current and proposed Australian Drinking Water Guidelines. |
| 5 |  | PFHxS (0.002) PFBS (0.001) | Detection below Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.008) PFHxS (0.039) PFOA (0.016) PFBS (0.031) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above proposed Australian Drinking Water Guidelines. |
| 9 |  | PFOS (0.017) PFHxS (0.134) PFOA (0.058) PFBS (0.106) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.035) PFHxS (0.168) PFOA (0.092) PFBS (0.138) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.013) PFHxS (0.155) PFOA (0.069) PFBS (0.146) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.06) PFHxS (0.565) PFOA (0.243) PFBS (0.383) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 6 | 8 May 2025 | PFOS (0.001) PFHxS (0.002) | Detection below Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 17 |  | N/D |  |
| 16 |  | N/D |  |
| 5 |  | PFHxS (0.001) | Detection below Australian Drinking Water Guidelines |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.006) PFHxS (0.022) PFOA (0.011) PFBS (0.011) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above proposed Australian Drinking Water Guidelines. |
| 9 |  | PFOS (0.009) PFHxS (0.093) PFOA (0.043) PFBS (0.045) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.016) PFHxS (0.084) PFOA (0.052) PFBS (0.038) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.009) PFHxS (0.088) PFOA (0.045) PFBS (0.047) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.057) PFHxS (0.366) PFOA (0.167) PFBS (0.187) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 6 | June 2025 | PFHxS (0.001) | Detection below Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 17 |  | N/D |  |
| 16 |  | N/D |  |
| 5 |  | N/D |  |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.004) PFHxS (0.015) PFOA (0.008) PFBS (0.007) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above proposed Australian Drinking Water Guidelines. |
| 9 |  | PFOS (0.009) PFHxS (0.093) PFOA (0.043) PFBS (0.045) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 10 |  | PFOS (0.016) PFHxS (0.084) PFOA (0.052) PFBS (0.038) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 11 |  | PFOS (0.009) PFHxS (0.088) PFOA (0.045) PFBS (0.047) | PFOA and PFBS below both current and proposed Australian Drinking Water Guidelines. PFOS and PFHxS above both current and proposed Guidelines. |
| 12 |  | PFOS (0.057) PFHxS (0.366) PFOA (0.167) PFBS (0.187) | PFBS below both current and proposed Australian Drinking Water Guidelines. PFOA below current guidelines only. PFOS and PFHxS above both current and proposed Guidelines. |
| 13 |  | N/D |  |
| 6 | 10 July 2025 | PFHxS (0.001) | Detection below Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 17 |  | N/D |  |
| 16 |  | N/D |  |
| 5 |  | N/D |  |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.005) PFOA (0.008) PFBS (0.015) | Detections below Australian Drinking Water Guidelines |
| 9 |  | PFOS (0.015) PFHxS (0.437) PFOA (0.178) PFBS (0.043) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above Guidelines. |
| 10 |  | PFOS (0.018) PFOA (0.051) PFBS (0.055) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS above Guidelines. |
| 11 |  | PFOS (0.013) PFHxS (0.905) PFOA (0.385) PFBS (1.04) | All above Australian Drinking Water Guidelines. |
| 12 |  | PFOS (0.026) PFHxS (1.76) PFOA (0.635) PFBS (1.67) | All above Australian Drinking Water Guidelines. |
| 13 |  | N/D |  |
| 6 | 7 August 2025 | PFHxS (0.001) | Detection below Australian Drinking Water Guidelines |
| 2 |  | N/D |  |
| 3 |  | N/D |  |
| 17 |  | N/D |  |
| 16 |  | N/D |  |
| 5 |  | N/D |  |
| 8 |  | N/D |  |
| 7 |  | PFOS (0.004) PFHxS (0.015) PFOA (0.007) PFBS (0.004) | Detections below Australian Drinking Water Guidelines |
| 9 |  | PFOS (0.023) PFHxS (0.107) PFOA (0.053) PFBS (0.02) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above Guidelines. |
| 10 |  | PFOS (0.019) PFHxS (0.073 PFOA (0.041) PFBS (0.015) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above Guidelines. |
| 11 |  | PFOS (0.028) PFHxS (0.109) PFOA (0.059) PFBS (0.025) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above Guidelines. |
| 12 |  | PFOS (0.05) PFHxS (0.166) PFOA (0.092) PFBS (0.029) | PFOA and PFBS below Australian Drinking Water Guidelines. PFOS and PFHxS above Guidelines. |
| 13 |  | N/D |  |